1. Build fix

- Namespace conflict (radar_display.cpp, runway_overlay.cpp, status_screens.cpp): removed a redundant namespace fonts = lgfx::v1::fonts; alias in each file — a newer LovyanGFX version already defines a global fonts namespace, so the manual alias was colliding with it.

2. Display hardware swap: round GC9A01 → TJCTM24024-SPI (ILI9341)

- config.h: new pin/size constants (kDisplayWidth/Height → 240×320, kDisplayPinBl for backlight, kDisplayInvert/kDisplayRgbOrder flipped for ILI9341).

- lgfx_config.hpp: Panel_GC9A01 → Panel_ILI9341, added Light_PWM backlight control.

- Fixed a leftover-yellow-background bug: the radar sprite only covers the top 240×240 square, so the new 80px strip below it wasn't being cleared — now cleared to black once on first frame (radar_display.cpp).

- README updated (wiring table, config table, hardware description).

3. New aircraft-select feature

- SELECT button (GPIO 6): new select_button.h/.cpp module; cycles through in-ring aircraft (with a "none" stop) via radarSelectionCycleNext().

- Target-lock ring: selected aircraft gets a white ring drawn around its symbol on the radar.

- Top-right range label: shows the outer-ring range (e.g. 16mi), reusing the existing formatRing3Label() formatter.

- Info panel (bottom strip, only on the taller display): callsign, ADS-B aircraft type, altitude, ground speed, plus:

  - Route lookup (flight_route.h/.cpp) — origin→destination airport codes and a friendlier country-municipality → country-municipality line (truncated with ... if too long), both from one api.adsbdb.com/v0/callsign/ call.

  - Aircraft registry lookup (aircraft_info.h/.cpp) — registered type/manufacturer, right-aligned in the panel, from api.adsbdb.com/v0/aircraft/{hex}.

  - Both lookups fire together, once, only when a new aircraft is selected — never on the periodic ADS-B poll.

  - Added Aircraft::hex field (adsb_client.h) since it was previously discarded once a callsign was known.

4. Stability fixes (the "unstable/frozen" investigation)

- Diagnostics: added GET/response timing and heap (free/max_alloc) logging to all three network calls (adsb_client.cpp, flight_route.cpp, aircraft_info.cpp) to actually see what was happening instead of guessing.

- Bug #1 — silent data loss on the ADS-B fetch: the old code accumulated the response into a String before parsing; under memory pressure, String::concat() was silently failing (return value never checked) so bytes were read off the socket and discarded without error, producing InvalidInput/IncompleteInput parse errors and 10s timeouts. Fixed by parsing directly off the HTTP stream via a small custom PollingStreamReader (no intermediate buffer), while still polling wifiLoop() during the read so the Wi-Fi portal stays responsive.

- Bug #2 — oversized JSON documents: adsb.fi sends ~20-30 raw fields per aircraft; the firmware only uses about a dozen. At larger ranges (more aircraft), storing all the unused fields exhausted the ~35KB available heap, causing slow reads, NoMemory errors, and once a TLS connection failure. Fixed with an ArduinoJson Filter that only parses/stores the fields actually used — everything else is skipped for free during parsing.

- Confirmed via logs: heap fully recovers to baseline between requests (no persistent leak), and the previously-stuck "EHAM" runway-label artifact was very likely a frozen frame from these fetch failures, not a separate bug — pending your retest to confirm it's resolved.

I also notice you've since changed the default radar coordinates in config.h (kDefaultRadarLat/Lon → 46.6390/7.0569) — that's just your own location update, unrelated to anything above.